Thank you for purchasing one of our quality Locash Racing products.

Product: 	OBD Conversion Harness

Model:		OBD0 to OBD2 ECU

# Part #:		OBD0-2A or OBD0-2B

In order to complete the installation of your conversion harness, additional sensors and/or equipment may need to be connected to the subharness.

Color			Function

## 9 Wire Pigtail

Black/White		Knock Sensor

Blue/White		Intake Air Bypass Power

Black/Green		Vtec Pressure Switch Control

Brown			Fuel Tank Pressure

Black/Orange		CKF-P

White/Yellow		CKF-M

White/Orange 		Service Check Signal

White/Green		Second O2 Sensor Signal

Black(ground) 		Service Check Ground

## 12 Wire Pigtail

Purple			Vtec Solenoid Signal

Blue			EGR Solenoid Valve

Green			O2 Sensor Heater Control

White			Intake Air Bypass Signal (P72/P13 ECUs only)

Orange			Alternator Control

Grey			EVAP Bypass Solenoid Valve

Red(power)		O2 Sensor Power

Yellow			EVAP Control Canister Vent

Pink			Second O2 Sensor Heater Control

Black(ground) 		Second O2 Sensor Ground

Black(ground) 		Vtec Pressure Switch Ground

Black(ground)		O2 Sensor Ground

### 4 Wire Pigtail

Red(power)		Second O2 Sensor Power

Black(ground) 		IAB Ground(Used in Some OBD2a applications)

Black/ Yellow		O2 Sensor Signal

Blue/Red		EGRL

Installation Notes:

IAB-  					Used only with P72 or P13 ECUs

Service Check (ground and signal)-  	These connections have been added for your convenience only, a simple toggle switch can be used to connect the two wires.  The use of this alternate service check connection is optional, if you choose to continue using the original system, simply cap the connections so as to not accidentally ground the signal wire and activate the service check circuit.

Powers and Grounds-	These are suggested connections,  other suitable locations may be used for powers and grounds.

IMPORTANT:

-The above connections may not be used in all configurations, JDM ECUs will not require most of the extra sensors.

-Please consult a professional installer in order to avoid serious damage to the vehicles components.

This product is designed for off road use only

Distributor Wiring Comparison:

OBD0				OBD2

Blue w/Green Stripe		Yellow

Blue w/Yellow Stripe		Black

Orange w/Blue Stripe		Green

White w/Blue Stripe		Red

Orange				Blue

White				White

Thick White			Yellow w/Green Stripe   (Ignitor Signal)

Alternator Wiring Comparison:

OBD0				OBD2

Yellow				White w/Red Stripe

White w/Yellow Stripe	White w/Green Stripe

Black w/Yellow Stripe	Black w/Yellow Stripe

White w/Blue Stripe		White w/Blue Stripe

If you are using OBD1 or OBD2 injectors, remove the injector resistor box

and connect all five wires that were originally going to the resistor

box together.

If you have any questions, please email info@locashracing.com